SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA0328 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | Manikanth goudar | Reg. No.: | 192311300 |
| Section / Batch: |  | Date: | 12-10-26 |

Code of Conduct I Manikanth goudar certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student:

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Questions

Q1. A telecom operator wins a spectrum auction for 45 MHz. Each simplex channel requires 25 kHz. How many full-duplex voice channels can be supported?

Answer: N = 45 × 1000 / (2 × 25) = 900 channels.

Q2. A regional carrier has 560 voice channels shared across a 7-cell cluster. How many channels does each cell receive?

Answer: N = 560 / 7 = 80 channels.

Q3. A GSM operator uses 100 carriers, each carrying 8 time slots. Find the total simultaneous voice channels.

Answer: N = 100 × 8 = 800 channels.

Q4. A base station transmits 80 W, while a handset transmits 4 W. Find the power difference in dB.

Answer: 10 log₁₀(80 / 4) = 13.01 dB.

Q5. A serving signal is −60 dBm and co-channel interference is −92 dBm. Find the C/I ratio.

Answer: C/I = −60 − (−92) = 32 dB.

Q6. An operator has 42 MHz of spectrum, with 300 kHz per channel, reused across 7 cells. Find the channels per cell.

Answer: N = 42,000 / (300 × 7) = 20 channels per cell.

Q7. A transmitter delivers 64 W. The receiver is moved twice as far away, with path-loss exponent 4. Find the new received power.

Answer: P₂ = 64 / 2⁴ = 4 W.

Q8. A cell's radius is doubled under an inverse-square power law. Find the required transmit-power factor to maintain the same SNR.

Answer: Power factor = 2² = 4 times.

Q9. A cluster has 4 cells, each with 15 voice channels. Find the total channels.

Answer: N = 4 × 15 = 60 channels.

Q10. A vehicle travels at 100 km/h on a 900 MHz carrier. Estimate the maximum Doppler shift.

Answer: fᵈ = vfᶜ/c = (27.78 × 900 × 10⁶) / (3 × 10⁸) ≈ 83.33 Hz.

Q11. A 5 ms TDMA frame has 8 equal time slots. Find one slot's duration.

Answer: T = 5 / 8 = 0.625 ms.

Q12. A 24 MHz spectrum allocation uses 200 kHz carrier spacing. Find the number of carriers.

Answer: N = 24,000 / 200 = 120 carriers.

Q13. A handset transmits 8 W and experiences 33 dB path loss. Find the received power in dBm.

Answer: Pₜ = 10 log₁₀(8000) = 39.03 dBm; Pᵣ = 39.03 − 33 = 6.03 dBm.

Q14. Find the free-space path loss at 1900 MHz over 2.5 km, using 1 m as the reference distance.

Answer: FSPL = 32.44 + 20 log₁₀(1900) + 20 log₁₀(2.5) ≈ 105.98 dB.

Q15. A cluster contains 6 cells, each handling 250 calls. Find the total capacity.

Answer: N = 6 × 250 = 1,500 calls.

Q16. A switching subsystem processes 4,000 calls per second, with an average call duration of 3 minutes. Find the number of active calls.

Answer: Active calls = 4,000 × 180 = 720,000 calls

Q17. A base station has 150 channels, each occupying 25 kHz. Find the total bandwidth.

Answer: B = 150 × 25 kHz = 3,750 kHz = 3.75 MHz.

Q18. A system uses 40 kHz guard bands between 30 adjacent frequency bands. Find the wasted bandwidth.

Answer: There are 29 gaps: B = 29 × 40 kHz = 1.16 MHz.

Q19. A vehicle travels at 90 km/h through cells of radius 1.5 km. Using the cell diameter as the crossing distance, estimate the handoffs per hour.

Answer: Cell diameter = 3 km; handoffs ≈ 90 / 3 = 30 per hour.

Q20. A network has 8 regional centers, each overseeing 40 base stations, with 250 users per base station. Find the total capacity.

Answer: N = 8 × 40 × 250 = 80,000 users.

Q21. Cover 400 km² using cells of radius 400 m, approximating each cell's area as R². Find the number of cells.

Answer: R = 0.4 km; cell area ≈ 0.4² = 0.16 km²; N = 400 / 0.16 = 2,500 cells.

Q22. An FDMA system allocates 30 kHz per channel from 6 MHz of spectrum. Find the number of channels.

Answer: N = 6,000 / 30 = 200 channels.

Q23. There are 1,200 subscribers, each making 4 calls per hour, averaging 2.5 minutes per call. Find the offered traffic in Erlangs.

Answer: A = 1,200 × 4 × (2.5 / 60) = 200 Erlangs.

Q24. A 40 MHz band allocates 400 kHz per connected user. Find the maximum simultaneous users.

Answer: N = 40,000 / 400 = 100 users.

Q25. A network has 1,200 base stations, each supporting 600 users. Find the total capacity.

Answer: N = 1,200 × 600 = 720,000 users.

Q26. A paging scheme searches 4 cells per round in a 200-cell network. Find the worst-case number of rounds.

Answer: N = 200 / 4 = 50 rounds.

Q27. A network supports 100,000 simultaneous connections and receives 40,000 call attempts per minute, each lasting 3 minutes. Estimate blocking.

Answer: Offered traffic = (40,000 / 60) × 3 = 2,000 Erlangs. This is far below 100,000 connections, so a simplified capacity comparison estimates 0% blocking. Exact blocking requires an Erlang B model and a defined number of traffic channels.

Q28. If 15% of 600,000 subscribers roam onto partner networks, how many subscribers roam?

Answer: N = 600,000 × 0.15 = 90,000 subscribers.

Q29. A switching center handles 9,000 calls per second out of a capacity of 12,000. Find the utilization.

Answer: Utilization = (9,000 / 12,000) × 100 = 75%.

Q30. A network has 48 MHz of spectrum, 200 kHz per channel, and a reuse factor of 4. Find the channels per cell.

Answer: N = 48,000 / (200 × 4) = 60 channels per cell.

Q31. A GSM deployment has 30 MHz of spectrum with 200 kHz carrier spacing. Find the number of channels.

Answer: N = 30,000 / 200 = 150 channels.

Q32. Out of 2,500 handoff attempts, 99% succeed. Find the successful handoffs.

Answer: N = 2,500 × 0.99 = 2,475 successful handoffs.

Q33. A handset transmits 12 W and experiences 45 dB path loss. Find the received power in dBm.

Answer: Pₜ = 10 log₁₀(12,000) = 40.79 dBm; Pᵣ = 40.79 − 45 = −4.21 dBm.

Q34. A 7-cell cluster supports 900 simultaneous users per cell. Find the total capacity.

Answer: N = 7 × 900 = 6,300 users.

Q35. A 4.8 ms TDMA frame contains 8 equal slots. Find each slot's duration.

Answer: T = 4.8 / 8 = 0.6 ms.

Q36. A mobile device travels at 150 km/h on a 1.8 GHz carrier. Find the maximum Doppler shift.

Answer: fᵈ = (41.67 × 1.8 × 10⁹) / (3 × 10⁸) ≈ 250 Hz.

Q37. A switch supports 250,000 simultaneous calls and receives 70,000 calls per minute, each lasting 4 minutes. Estimate blocking.

Answer: Offered traffic = (70,000 / 60) × 4 ≈ 4,666.67 Erlangs. This is below 250,000 connections, so a simplified capacity comparison estimates 0% blocking. Exact blocking requires an Erlang B model and a defined number of traffic channels.

Q38. A cluster contains 9 cells, each supporting 600 users. Find the total capacity.

Answer: N = 9 × 600 = 5,400 users.

Q39. A switching center processes 60,000 calls per second, with 0.4% dropped. Find the number of dropped calls per second.

Answer: Dropped calls = 60,000 × 0.004 = 240 calls per second.

Q40. A base station transmits at 43 dBm and receives a signal of −87 dBm. Find the path loss.

Answer: Path loss = 43 − (−87) = 130 dB.

Q41. Each base station covers 2.5 km². Find the total area covered by 150 stations.

Answer: Area = 2.5 × 150 = 375 km².

Q42. A 48 MHz allocation uses 200 kHz channels with a reuse factor of 6. Find the channels per cell.

Answer: N = 48,000 / (200 × 6) = 40 channels per cell.

Q43. A paging system processes 6,000 users per cycle in a network of 150,000 users. Find the number of cycles.

Answer: N = 150,000 / 6,000 = 25 cycles.

Q44. A call lasts 9 minutes, with a handoff approximately every 3 minutes. Estimate the number of handoffs.

Answer: Estimated handoffs = 9 / 3 = 3

Q45. A signal is −85 dBm and the noise floor is −115 dBm. Find the SNR.

Answer: SNR = −85 − (−115) = 30 dB.

Q46. A 4G cell supports 250 users, each requiring 150 kbps. Find the minimum bandwidth assuming spectral efficiency of 1 bit/s/Hz.

Answer: Total rate = 250 × 150 kbps = 37.5 Mbps. At 1 bit/s/Hz, minimum bandwidth = 37.5 MHz.

Q47. A base station serves 1,500 subscribers, each making 4 calls per hour with a 2.5-minute average duration. Find the offered traffic.

Answer: A = 1,500 × 4 × (2.5 / 60) = 250 Erlangs.

Q48. A network reserves 250 control channels, each 12 kHz wide. Find the total bandwidth.

Answer: B = 250 × 12 kHz = 3,000 kHz = 3 MHz.

Q49. A network carries 1,500 simultaneous connections, each requiring 600 kbps. Find the total throughput.

Answer: Throughput = 1,500 × 600 kbps = 900,000 kbps = 900 Mbps.

Q50. Towers cover a circular area with a 4 km radius. Find the number of towers required to cover 350 km².

Answer: Area per tower = π × 4² ≈ 50.27 km²; towers = 350 / 50.27 ≈ 6.96, so round up to 7 towers